实验二 通过rqt查看机器人的数据结构

实验类型：演示/实践/仿真       实验学时：2小时        实验要求：必修

一、实验目的

1、掌握查看Aimibot教育机器人数据结构的方法；

2、熟练编写ROS话题发布者与订阅者的程序。

二、实验内容

1、通过 ROS 的可视化查看数据类型与数值操作。

三、实验器材

1、硬件：Aimibot教育机器人主体、键盘鼠标。

2、软件：Ubuntu16.04 + ROS

四、实验原理

1、话题通信原理

2、程序代码

- 打开Amos_WS/aimibotV2文件夹，可以看到lunch、src、include等多个文件夹，其中test.launch文件位于lunch文件夹中，aimibot.h文件位于include文件夹中，main.cpp文件位于src文件夹中，依次打开三个文件，查看代码。

- test.launch文件

- 打开Amos_WS/aimibotV2/lunch/test.launch文件，该lunch文件运行了aimibot可执行文件，其中对aimibot可执行文件中的参数进行赋值，该函数代码如下：

<launch> <arg name="serialport" default="/dev/ttyUSB0" /> //串口使用USB0 <node pkg="aimibot" type="aimibot" name="aimibot" output="screen"> <param name="serial_port" value="$(arg serialport)"/> <param name="odom_frame" value="odom"/> <param name="base_frame" value="base_footprint"/> <param name="use_imu_heading" value="true" /> <param name="publish_tf" value="true" /> </node> </launch>

- aimibot.h文件

打开Amos_WS/aimibotV2/include/aimibot.h文件，该头文件创建了如下话题：

/aimibot/CoreSensors：发布控制系统核心数据，包括防跌落传感器(cliff)、左/右轮编码器(left_encoder:65364,right_encoder:65234)、左/右轮电机接收的 PWM 值 (left_pwm,right_pwm，暂无数据)、外部按键、电池电压（battery:例如 17.64）、输出电流；

/aimibot/GpInput：发布检测到的外部接口的输入值（analog_input,digtal_input（模拟信号和数字信号）

/aimibot/Ultrasonic：发布超声波传感器数值(disl1~disl6：数值大概在 0-150)

/aimibot/cliff：发布防跌落传感器数值: bottom（3，0，3）

/aimibot/controller_info：发布机器人的内部信息（d_gain,i_gain,p_gain,type）

/aimibot/current：发布电源输出电流（无）

/aimibot/sensors/dock_ir：发布红外传感器数据（docking,暂无）

/aimibot/sensors/imu_data：发布陀螺仪数据（三轴加速度与机器人的偏转角度和角速度）

/aimibot/sensors/imu_data_raw

/aimibot/unique_id：发布机器人特有的 ID 识别号

/aimibot/version_info：发布机器人的版本信息

/joint_states：发布机器人的驱动电机状态

/odom：发布里程计信息

并创建了对应的发布者与订阅者，函数如下：

void subscribeTopics(ros::NodeHandle& nh) { velocity_command_subscriber=nh.subscribe<geometry_msgs::Twist>("/aimibot/commands/velocity", 10, &subscribeVelocityCommand); led1_command_subscriber=nh.subscribe<aimibot::Led>("/aimibot/commands/led_1", 10, &subscribeLed1Command); led2_command_subscriber=nh.subscribe<aimibot::Led>("/aimibot/commands/led_2", 10, &subscribeLed2Command); digital_output_command_subscriber=nh.subscribe<aimibot::DigitalOutput>("/aimibot/commands/digi_output", 10, &subscribeDigitalOutputCommand); external_power_command_subscriber=nh.subscribe<aimibot::DigitalOutput>("/aimibot/commands/exter_power", 10, &subscribeExternalPowerCommand); sound_command_subscriber=nh.subscribe<aimibot::Sound>("/aimibot/commands/beep_sound", 10, &subscribeSoundCommand); reset_odometry_subscriber=nh.subscribe<std_msgs::Empty>("/aimibot/commands/reset_odometry", 10, &subscribeResetOdometry); motor_power_subscriber=nh.subscribe<aimibot::MotorPower>("/aimibot/commands/mot_power", 10, &subscribeMotorPower); controller_info_command_subscriber=nh.subscribe<aimibot::ControllerInfo>("/aimibot/commands/control_info", 10, &subscribeControllerInfoCommand); } void advertiseTopics(ros::NodeHandle& nh) { cliff_state_publisher = nh.advertise <aimibot::Cliff>("/aimibot/cliff",100); joint_state_publisher = nh.advertise <aimibot::CoreSensors>("/aimibot/CoreSensors",100); version_info_publisher = nh.advertise < aimibot::hardware > ("/aimibot/version_info", 100, true); // latched publisher controller_info_publisher = nh.advertise < aimibot::ControllerInfo > ("/aimibot/controller_info", 100, true); // latched publisher dock_ir_publisher = nh.advertise < aimibot::DockInfraRed > ("/aimibot/sensors/dock_ir", 100); inertia_publisher = nh.advertise < aimibot::Inertia > ("/aimibot/sensors/imu_data", 100); raw_imu_data_publisher = nh.advertise < aimibot::ThreeAxisGyro > ("/aimibot/sensors/imu_data_raw", 100); unique_id_publisher = nh.advertise< aimibot::UniqueDeviceID > ("/aimibot/unique_id", 100); current_publisher = nh.advertise< aimibot::Current > ("/aimibot/current", 100); gpinput_publisher = nh.advertise< aimibot::GpInput > ("/aimibot/GpInput", 100); ultrasonic_data_publisher = nh.advertise< aimibot::Ultrasonic > ("/aimibot/Ultrasonic", 100); }

- main.cpp文件

- 打开Amos_WS/aimibotV2/src/main.cpp文件，

- 函数入下：

int main(int argc, char** argv) { ros::init(argc, argv, "serial_port"); //ROS初始化 ros::NodeHandle nh; //创建句柄 ros::NodeHandle nh_p("~"); signal(SIGINT, shutdown); //按下Ctrl+c关闭串口 advertiseTopics(nh); //创建发布者 subscribeTopics(nh); //创建订阅者 ros::param::get("~serial_port", port); //获取串口 try //配置串口 { serial::Timeout to = serial::Timeout::simpleTimeout(1000); //创建timeout sp.setPort(port); //设置要打开的串口名 sp.setBaudrate(115200); //设置串口通信的波特率 sp.setTimeout(to); //串口设置timeout sp.open(); //打开串口 } catch(serial::IOException& e) //获取串口异常信息 { ROS_ERROR_STREAM("Unable to open port."); return -1; } //判断串口是否打开成功 if(sp.isOpen()) ROS_INFO_STREAM("Device is opened."); else return -1; ros::Rate loop_rate(100); //设置循环频率 ros::Time last_getdata = ros::Time::now(); //获取当前时间 uint8_t temp_buffer[256]; uint8_t buffer[256]; uint8_t i =0; ros::Publisher joint_state_publisher=nh.advertise <sensor_msgs::JointState>("/joint_states",100); //创建驱动电机状态的发布者 ecl::Angle<double> gethand; sensor_msgs::JointState joint_states; ecl::LegacyPose2D<double> pose_update; ecl::linear_algebra::Vector3d pose_update_rates; joint_states.name.push_back("leftwheel_joint"); joint_states.name.push_back("rightwheel_joint"); while(ros::ok()) { size_t n = sp.read(temp_buffer, ready_get); //获取缓冲区内的字节数 if(n==0) //串口没有传输数据 { std::cout<<"waiting for data 10s" << std::endl; if(ros::Time::now() - last_getdata > ros::Duration(10.0)) //等待超时 { std::cout<< "TIME OUT!CLOSE SERIAL!"<<std::endl; sp.close(); //关闭串口 ros::shutdown(); //关闭ROS }continue; } if(findpack_update(temp_buffer,n)) //读到串口数据 { if(!head_init) { heading_offset = init_head(); head_init = true;} last_getdata=ros::Time::now(); //获取当前时间 rostopic_pub(coresensors_data, ultrasonic_data, gpin_data, threeAG_data, firmware_Data, hardware_data, current_data, cliff_Data, iner_data, dockin_data); //发布话题 joint_states.position.resize(2,0.0); //初始化joint_states默认值 joint_states.velocity.resize(2,0.0); joint_states.effort.resize(2,0.0); if(ros::ok()) { joint_states.header.stamp = ros::Time::now(); joint_state_publisher.publish(joint_states); //发布驱动电机状态 } } ros::spinOnce(); } //关闭串口 sp.close(); return 0; }

五、实验步骤

1、获取串口操作权限（已映射），详见说明书第X章。

2、打开终端，启动test.launch文件，执行：

roslaunch aimibot test.launch

3、另开终端执行：

rosrun rqt_topic rqt_topic

如图所示：

六、实验注意事项

七、预习与思考题

八、进阶实验